### Networking Project 2

### Mission 1

Determine and document the mail servers for starwars.com using NSLOOKUP.

Explain why the Resistance isn't receiving any emails.

The reason the Resistance isn’t receiving any emails is because an intruder has changed the primary and secondary mail servers.  The correct primary and secondary mail servers should be: asltx.1.google.com and asltx.2.google.com

Document what a corrected DNS record should be.

The correct DNS record should be:

starwars.com	mail exchanger = 1 asltx.1.google.com.

starwars.com	mail exchanger = 5 asltx.2.google.com.

### Mission 2

Determine and document the SPF for theforce.net using NSLOOKUP.

theforce.net	text = "v=spf1 a mx mx:smtp.secureserver.net include:aspmx.googlemail.com ip4:104.156.250.80 ip4:45.63.15.159 ip4:45.63.4.215"

Explain why the Force's emails are going to spam.

While the network was down The Force changed their IP address mail server to 45.23.176.21 and it is not matching the SPF record.

Document what a corrected DNS record should be.

The correct DNS record should include IP address 45.23.176.21

### Mission 3

Document how a CNAME should look by viewing the CNAME of www.theforce.net using NSLOOKUP.

Explain why the sub page of resistance.theforce.net isn't redirecting to theforce.net.

The CNAME is missing a reference to direct “resistance.theforce.net” to “theforce.net”.

Document what a corrected DNS record should be.

The correct DNS record should be:  www.resistance.theforce.net

### Mission 4

Confirm the DNS records for princessleia.site.
The DNS record for princessleia.site does not have ns2.galaxybackup.com listed as a nameserver.

Document how you would fix the DNS record to prevent this issue from happening again.

To prevent this issue from happening again ns2.galaxybackup.com should be added as a nameserver.

Non-authoritative answer:

princessleia.site	nameserver = ns26.domaincontrol.com.

princessleia.site	nameserver = ns25.domaincontrol.com.

princessleia.site	nameserver = ns2.galaxybackup.com.

Mission 5

View the Galaxy Network Map and determine the OSPF shortest path from Batuu to Jedha.

Batuu-D-C-E-F-J-I-L-Q-T-V-Jedha

Confirm your path doesn't include Planet N in its route.

The route does not include “N”.

Document this shortest path so it can be used by the Resistance to develop a static route to improve the traffic.

Batuu-D-C-E-F-J-I-L-Q-T-V-Jedha

1-2-1-1-1-1-6-4-2-2-2 = 23 hops.

### Mission 6

Figure out the Dark Side's secret wireless key by using Aircrack-ng.

aircrack-ng -w /usr/share/wordlists/rockyou.txt Darkside.pcap

Hint: This is a more challenging encrypted wireless traffic using WPA.
In order to decrypt, you will need to use a wordlist (-w) such as rockyou.txt.

Use the Dark Side's key to decrypt the wireless traffic in Wireshark.

Once you have decrypted the traffic, figure out the following Dark Side information:
Host IP Addresses and MAC Addresses by looking at the decrypted ARP traffic.
Document these IP and MAC Addresses, as the resistance will use these IP addresses to launch a retaliatory attack.

### Mission 7

View the DNS record from Mission #4.

The Resistance provided you with a hidden message in the TXT record, with several steps to follow.

Follow the steps from the TXT record.
Note: A backup option is provided in the TXT record (as a website) in case the main telnet site is unavailable

Take a screenshot of the results.
sysadmin@UbuntuDesktop:~$ telnet towel.blinkenlights.nl  worked on the command line but I could not stop it to take a screenshot.  So I used the website www.asciimation.co.nz

### Conclusion

- Submit your results and findings from every mission.

- Congratulations, you have completed your mission and saved the Galaxy!